Fluffy Programming Protocol

3rd March 1999

Disclaimer:

This document is intended as a guide for those wishing to write their own software for use with the Fluffy SX programmer. All the information included is believed to be correct at the time of writing, but I accept no responsibility for any damage caused, direct or indirect, by the use of this document. This guide is not guaranteed to be complete - you are strongly encouraged to study the source code (fluffy.c and sxprog.asm).

Overview:

This document describes the protocol used for communications between the PC, and the 16F84 that controls the Fluffy programmer. This communication can take place via the PC serial port (RS232), or via the PC parallel port. The protocol itself is more or less identical, regardless of which physical interface is used. All communication between the PC and Fluffy occurs in small synchronous packets. In each packet, 4 bytes of data are transferred from the PC to Fluffy, and 2 bytes of data are transferred from Fluffy to the PC.

You will probably notice that most of the complex functionality involved in programming a SX chip is the responsibility of the PC software. The programmer itself provides only a thin wrapper between the SX and the PC, and does not attempt to simplify or hide the SX programming protocol. The reason for this is that it is far easier to update the PC software than it is to update the code in the 16F84. The main purpose of the 16F84 is to take care of the sensitive timing issues involved in programming an SX chip.

Parallel Interface:

The parallel interface uses 4 data + 1 control line (ACK) from the PC to the 16F84, and 2 data + 2 control lines (FIRST, REQ) from the 16F84 to the PC. The entire packet is transmitted in 8 “chunks”, each chunk involving the transfer of 4 bits from the PC to the 16F84, and 2 bits from the 16F84 to the PC.

D0-D3 are the 4 data lines from the PC to the 16F84. FIRST and REQ are control lines from 16F84 to PC. R0-R1 are the 2 data lines from the 16F84 to the PC, and ACK is the single control line from the PC to the 16F84. (Note that these names are for convenience only, and do not necessarily correspond to the pin names on the parallel port).

The sequence of events is as follows, and is initiated by the 16F84. Steps in square brackets [] are performed only on the first “chunk” of the sequence. This allows the PC to ensure that everything is in sync.

- 16F84 places 2 bits of information on R0-R1.

- [16F84 raises FIRST, to signal that this “chunk” is the first in the sequence.]

- 16F84 raises REQ to signal that data is ready.

- PC detects change in REQ, and reads R0-R1 [also observes FIRST].

- PC places 4 bits of its data on D0-D3.

- PC raises ACK to signal that data is ready.

- 16F84 detects change in ACK, and reads D0-D3.

- [16F84 lowers FIRST.]

- 16F84 places 2 bits of information on R0-R1.

- 16F84 lowers REQ to signal that data is ready.

- PC detects change in REQ, and reads R0-R1.

- PC places 4 bits of data on D0-D3.

- PC lowers ACK to signal that data is ready.

- 16F84 detects change in ACK, and reads D0-D3.

This process is repeated 4 times (resulting in the total transfer of 32 bits from the PC, and 16 bits from the 16F84).

When using the parallel interface, the PC also has control of the 16F84’s /MCLR line, which is pulsed at the very start of the program, in order to ensure that the 16F84 is in a known state.

The physical pin connections are:

Signal			PC Parallel Port Name (pin no)

FIRST			Ack (10)

REQ			Paper (12)

ACK			D4 (6)

D0			D0 (2)

D1			D1 (3)

D2			D2 (4)

D3			D3 (5)

R0			Error (15)

R1			Sel In (13)

MCLR			D5 (7)

Serial Interface:

The serial interface runs at 38.4Kbps, and uses 8 data bits, no parity, and 1 stop bit. When the 16F84 powers up, it will be listening on the parallel interface only. However, if a certain number of transitions on the D0 line are detected, without any corresponding transitions on the ACK line, then the 16F84 will switch into serial mode, and start listening for serial signals on the D0 line. From then on, the 16F84 will only return to parallel mode via a reset (since the parallel interface allows the PC to control the 16F84’s /MCLR line, this is not a problem). Upon switching to serial mode, the first thing the 16F84 will do is transmit a series of ‘!’ symbols. This is a signal to the PC that serial mode has been entered.

The only added complication is that the serial interface does not allow the PC to reset the 16F84 to a known state. The PC does not know, when the program starts, whether or not the 16F84 is already in serial mode. Therefore, the PC starts by issuing repeated 09h characters. Instruction 09h is the RESET instruction. If the 16F84 is already in serial mode, it will reset itself (by jumping to 0000h), and thus come up in parallel mode. Once in parallel mode, the repeated transitions on the D0 line will cause it to switch into serial mode. Of course, if the 16F84 was originally in parallel mode, the switch to serial will occur anyway.

Each packet in serial mode is initiated by a ‘*’ character from the 16F84. The exchange of data is synchronous - the PC and 16F84 each wait for a character from the other, before sending further data. This eliminates the need for hardware handshaking, and consequently reduces the number of required connections between the PC and the 16F84.

16F84				PC

‘*’		->

<-		D0-D7

R0-R7		->

<-		D8-D15

R8-R15	->

<-		D16-D23

CHK1		->

<-		D24-D31

As with the parallel mode, 32 bits of data (4 bytes) are sent from the PC to the 16F84, and 16 bits of data (2 bytes) are transferred in the opposite direction. The pseudo-code used in fluffy.c to initialise the 16F84 in serial mode is something along the lines of:

Initialise serial port 38400,8,N

Send 09h characters, until a ! character is received

Keep reading characters, until the !’s stop, and a * arrives

Send 00h (nop), wait for reply

Send 00h, wait for reply

Send 00h, wait for reply

Send 00h

MainLoop:	Read a character. If it’s not a *, something is wrong.

Exchange packet

Goto MainLoop

The nop command is optional, but allows the PC to ensure that the 16F84 is exactly at the start of a packet (rather than possibly having more ! characters to send). The actual code also contains timeout/error checks.

Protocol

As stated previously, the protocol itself is the same, regardless of which physical interface is used. Each 32-bit packet from the PC represents a command, and the 16-bit reply from the 16F84 represents a response, from the previous command. Typically, the 16-bit reply will contain an error code, and/or some numerical result depending on the previous command issued.

The first byte of the 32-bit packet is always the actual command code itself, and the remaining 3 bytes make up the arguments. The commands are as follows:

Command	Format					Response

NOP		00	??	??	??		00	00

Do nothing.

OSC1		01	??	??	Level		00	00

Set the OSC1 output to 0V, 5V or 12.5V, according to Level (00, 01 or 02 respectively).

OSC2		02	??	??	Level		00	00

Set the OSC2 output to 0V or tristate, according to Level (00 or 01 respectively).

ECHO		03	??	?A	BC		0A	BC

Echo the low 12-bits of the data provided back to the PC. Used to test communications.

SXCMD	04	REPT	HH	LL		ER	RR

Issue a command to the SX chip, (assuming that ISP mode has already been entered by manipulation of OSC1 and OSC2). REPT specifies a repeat count (0 implies 256 repeats). HHLL represent the 16 bits of data to be transmitted to the SX. In the case of, for instance, a read command, the bits where the SX is expected to drive the OSC2 line should be set to 1, to avoid conflicts. (For a list of SX commands and responses, you should refer to the SX User Guide, available from www.scenix.com). The 16-bits returned are of the form ERRR, where E is a 4-bit error code, and R is the 12-bit response from the SX chip. The error codes have the following meaning:

0		OK

2		Unexpected bit count while communicating with SX (SX comms error)

3		Timeout waiting for 128khz programming signal (SX comms error)

PICINIT	05	??	??	??		00	00

Prepare to program a 16F84. This will drive the B6/B7 (clock/data) PIC programming lines low.

PICCMD	06	CMD	??	??		00	00

Send a short (6-bit) command to the PIC.

PICCMDD	07	CMD	HH	LL		hh	ll

Send a long (6-bit cmd + 16-bit data) command to the PIC. In the case of, for instance, a read command, the bits where the PIC is expected to drive the data line should be set to 1, to avoid conflicts. The 16-bits returned, hhll, represent the response from the PIC.

PICCLOSE	08	??	??	??		00	00

Return the B6/B7 (clock/data) PIC programming lines to tristate.

RESET		09	??	??	??		??	??

Reset the 16F84.